Article: IoT and Machine Learning Based Energy usage and Energy Demand Forecasting System

Abdulazeez Ilyasu

Abstract: Energy usage and energy demand forecasting is an essential process in electric power system operation, maintenance and planning. It involves the accurate prediction of both magnitudes and geographical locations of electric load over the different periods of the planning horizon. Many economic implications of power utility such as economic scheduling of generating capacity, scheduling of fuel purchases, security analysis, planning of power development, maintenance scheduling and dispatching of generation units are mainly operated based on accurate load forecasting. This paper presents the IoT and machine learning model of electricity demand forecasting for power system management. The introductory section provides the importance of electricity demand forecasting. Subsequent sections cover recent trends of demand forecasting techniques development in energy power systems. Several models have been surveyed to identify the demand pattern and predict the future demand. These techniques would be useful to determine the powerful energy management strategy so as to meet the required load demand at minimum operating cost.

- Introduction

With the globally increasing electricity demand projecting to increase by more than 85% in 2040 as living standards rise, economies expand and the need for electrification of society continues [1]. Technological advancements are changing the shape of the grid by converting a demand driven power system towards a generation power driven system. This is essentially due to a multitude of factors including increased the penetration of renewable energy resources (RES) and new technologies at consumer side. This creates uncertainties in terms of the future electricity demand. Therefore, the importance of load forecasting has increased multifold for the future in global electricity demand is consistent and still expected to grow more than twice the rate of the global energy demand. Forecasting in the energy sector is a crucial input for many power system applications of both a technical and managerial nature. These applications range from energy generation (from different renewable and non-renewable resources), energy management at different nodes and sectors, energy pricing and many others. The primary purpose of having a forecasted load for these application is to ensure a safe, reliable and affordable energy supply. Electricity demand forecasting plays an important role in load allocation and planning for future generation facilities and transmission augmentation [2]. Load demand in a given season is subject to a range of uncertainties, including underlying population growth, climate change and economic conditions. In addition, historical data are of importance in demand forecasting. Parallel to the increased demand of Electric power forecasting solutions, the means for forecasting with the help of IoT and Machine Learning techniques have in recent years gained in popularity relative to traditional time series predictive models. Although IoT and Machine Learning techniques are nothing new, the improved computational capacity and the higher availability of quality data have made the techniques useful for forecasting.

- Problem Statement

The changing economic structure due to industrial activities and consequent rapid urbanization of these economies add another dimension to the economic transition where a growing urban sector co-exists with a predominantly rural economy. The electrical demand arises from complex interactions between multiple personal, corporative and socio-economic factors. All these sources make power demand forecasting difficult. Indeed, an ideal model able to forecast the power demand with the highest possible level of accuracy would require access to virtually infinite data sources in order to feed such a model with all the relevant information.

- Objectives

The main objective of this research is to benchmark different forecasting techniques of electrical energy output. Towards this end, IoT and Machine Learning techniques can be used to dynamically learn the energy usage and the demand forecasting of the electrical energy output systems. IoT and Machine Learning techniques are benchmarked to traditional time series methods on energy system data from existing installations. This also required an investigation of feature engineering methodologies, which can be used to increase the overall prediction accuracy.

- Preliminary Literature Review

In this chapter, general articles related to energy forecasting is demonstrated. Significant findings for time series and machine learning techniques for energy forecasting is also presented.

Hong et al. [8] provide an outline of various forecasting techniques used for energy forecasting. The study emphasizes that many different time series and Machine Learning techniques have been used in various energy contexts. The study is based on a competition for energy forecasting and evaluates the methods used in the competition (Global Energy Forecasting Competition 2014).

Kestylev et al. [9] outlines a summary of various techniques employed in the field of solar PV power forecasting. The authors propose that the industry should establish an industry standard. Some useful findings, which are observed in other studies, are for instance substantial improvements for long-term forecasting when using numerical weather predictions (NWP) as input data. Furthermore, modeling future cloud positions with satellite-based data have improved short-term solar PV energy output forecasting. Finally, model accuracy tends to vary depending on the climatic condition of the forecasting location. As of this, a model is likely to perform better in one region than when trained on multiple sites simultaneously. Similarly, as climatic conditions can vary over a yearly cycle, a model may perform better when trained on one weather season rather than several.

- Methodology

Based on their forecasting horizons, electricity load forecasts can be broadly categorized into three distinct categories. These categories are: Short term load forecasting (STLF), Medium term load forecasting (MTLF) and Long term load forecasting (LTLF) [3].

STLF is usually carried out over time periods ranging from hours to days or weeks ahead. It helps in facilitating electricity markets for the day ahead planning of the electricity supply, and in demand side management as well. MTLF, on other hand, deals with the forecasting horizons of months to even years ahead. Such forecasts help in revenue assessments, unit maintenance scheduling, and energy trading etc. For LTLF, the forecast horizon stretches from roughly five years to even decades ahead. These forecasts provide a deeper insight for policy makers and help with the efficient management of assets and effective power systems expansion planning.

- Factor affecting electricity demand forecasting

The aims of load forecasting are to predict the load pattern. There are several factors that should be taken into consideration for load forecasting, which can be classified as time factor, economic factor, weather condition and customer factor [10-12].

Time factor: Sudden changes of weather for STLF, seasonal climate change for MTLF, change of gross national product and the population for LTLF. Weather condition: Temperature, Humidity, Wind speed and cloud cover. Economic factor: Industry development, Population growth, Cost of electricity, Customer factor: Residential consumer, commercial consumer and industrial consumer. The residential load curve is somewhat different from commercial and industrial customers.

- Mathematical model

Electricity demand forecasting techniques can be classified as follows [13 -15]:

- Multiple regression

- Exponential smoothing

- Iterative reweighted least-squares

- Adaptive load forecasting

- Stochastic time series

- Neural networks

- Conclusion

Accurate load forecasting is very important for electric Utilities. At the end of this research we reviewed some statistical and artificial intelligence techniques that are used for electric load forecasting. We discussed factors that affect the accuracy of the forecasts such as weather data, time factors, customer classes, as well as economic factors. How advanced mathematical modeling methods will be used for Load forecasting, and how additional progress in load forecasting and its use in industrial applications can be achieved by providing short-term load forecasts.

References

[1] IEA. World Energy Outlook 2015, International Energy Agency, Paris, France, 2015.

Pessanha, J.F.M., Leon, N. Forecasting Long-term

[2] Electricity Demand in the Residential Sector. Procedia Computer Science. 2015, 55, 529-38.

[3] Al-Alawi, S.M.; Islam, S.M. Principles of electricity demand forecasting. Part 1: Methodologies. Power Eng. J. 1996, 11, 91–95.

[4] Suganthi, L.; Samuel, A.A. Energy models for demand forecasting-A review. Renew. Sustain. Energy Rev.2012, 16, 1223–1240.

[5] Hernandez, L.; Baladron, C.; Aguiar, J.M.; Carro, B.; Sanchez-Esguevillas, A.J.; Lloret, J.; Massana, J. A survey on electric power demand forecasting: Future trends in smart grids, microgrids and smart buildings. IEEE Commun. Surv. Tutor. 2014, 16, 1460–1495.

[6] Khan, Z.A.; Jayaweera, D. Approach for forecasting smart customer demand with significant energy demand variability. In Proceedings of the 2018 1st International Conference on Power, Energy and Smart Grid (ICPESG), Mirpur Azad Kashmir, Pakistan, 9–10 April 2018; pp. 1–5.

[7] Rich H. Inman, Hugo T.C. Pedro, and Carlos F.M. Coimbra. Solar forecasting methods for renewable energy integration. Progress in Energy and Combustion Science, 39(6):535 – 576, 2013.

[8] Tao Hong, Pierre Pinson, Shu Fan, Hamidreza Zareipour, Alberto Troccoli, and Rob J. Hyndman. Probabilistic energy forecasting: Global energy forecasting competition 2014 and beyond. International Journal of Forecasting, 32(3):896 – 913, 2016.

[9] V Kostylev, A Pavlovski, et al. Solar power forecasting performance–towards industry standards. In 1st international workshop on the integration of solar power into power systems, Aarhus, Denmark, 2011.

[10] Clements, A.E., Hurn, A.S., Li, Z. Forecasting dayahead electricity load using a multiple equation time series approach. European Journal of Operational Research. 2016, 251, 522-30.

[11] Hong, T., Fan, S. Probabilistic electric load forecasting: A tutorial review. International Journal of Forecasting. 2016.

[12] Xiao, L., Shao, W., Liang, T., Wang, C. A combined model based on multiple seasonal patterns and modified firefly algorithm for electrical load forecasting. Applied Energy. 2016, 167, 135-53.

[13] Almeshaiei, E., Soltan, H. A methodology for Electric Power Load Forecasting. Alexandria Engineering Journal. 2011, 50, 137-44.

[14]Alfares, H.K., Nazeeruddin, M. Electric load forecasting: literature survey and classification of methods. International Journal of System Science. 2002, 33, 23-34.

[15]Soliman, S.A., Ahmad, M.A. Electrical Load Forecasting: Modeling and Model Construction, Elsevier, 2010.